| • 系統編號 | RC8901-0122 |  |  |
| --- | --- | --- | --- |
| • 計畫中文名稱 | 吸收性硬組織再生材料之研發 |  |  |
| • 計畫英文名稱 | Develop and Search for a New Biomaterial as Bioabsorbable Hard Tissue Regenerative Material |  |  |
| • 主管機關 | 行政院國家科學委員會 | • 計畫編號 | NSC88-2213-E038-001 |
| • 執行機構 | 台北醫學院醫學研究所 |  |  |
| • 本期期間 | 8708 ~ 8807 |  |  |
| • 報告頁數 | 0 頁 | • 使用語言 | 中文 |
| • 研究人員 | 曾厚 Tseng, How |  |  |
| • 中文關鍵字 | 生醫材料；氫氧磷灰石；齒槽骨；膠原蛋白；細胞毒性 |  |  |
| • 英文關鍵字 | Biomaterial；Hydroxyapatite；Alveolar bone；Collagen；Cytotoxicity |  |  |
| • 中文摘要 | 本研究利用燒結的方法製作多孔性的氫氧磷灰石(HA)燒結體,加入聚乳酸及由高分子量聚乳酸降解的乳酸寡聚物的混合物製作成PLLA/HA複合材,以做為硬組織生物可吸收性修補材之用,並對所得的PLLA/HA複合材進行其各種性質的探討。由實驗結果中可知,使用砂糖當作填充劑,以HA比砂糖為1比1.4, 25-45kgf/cm/sup 2/的壓力壓製成形,再以220.degree.C減壓下加熱使砂糖溶解,並且在1340.degree.C燒結4-6小時的燒結物的強度以及孔隙度為最佳。在乳酸寡聚物的製備上,發現高分子聚乳酸會隨著酸的濃度以及降解時間而分子量降低,分子鏈數增加,結果可由黏度值、DSC、FT-IR、X-ray及接觸角的測定證實。從毒性測試的結果可知未洗過的HA對細胞造成的毒性最高,而使用氯仿洗過的HA毒性最低。 |  |  |
| • 英文摘要 | The porously sintered hydroxyapatite (HA) and polylactide (PLLA) oligomer were used to prepare a series porous poly(L-lactide) /hydroxyapatite (PLLA/HA) composites as a biodegradable hard tissue repair material, and its physical-chemical properties were studied in this investigation. The used PLLA oligomer, mentioned above, was prepared via acidic hydrolytic reaction. The HA sinter with optimal strength and porosity could be obtained when HA was mixed with 1.4 of fold sucrose which was used as filler while binder, pressed under 25-45kgf/cm/sup 2/ force, heated at 220.degree.C to led to a sucrose melting under diminished pressure, and then sintered under 1340.degree.C for 4-6h. The all results of reduced viscosity, thermal properties, IR spectra, X-ray diffraction pattern, and contact angle wholly reveals that the PLLA molecular weight is decreased and its molecular number is also increased with the acid amount and degradation time increasing. HA without any purification have a highest toxicity and HA washed with chloroform have a lowest toxicity against gingival fibroblast were found. |  |  |