台北市乾洗業界使用石油系乾洗劑化學成份之初探

A Preliminary Chemical Study on the Petroleum Series Dry Cleaning Agents used in Dry cleaners in Taipei

中文摘要

長期暴露於高濃度石油系乾洗溶劑下會造成

皮膚炎與中樞神經抑制，如果含高濃度苯，則有

引發再生不良性貧血及白血病的可能；如國內市

售汽油中含有2%的苯。本研究收集台北市乾洗業

界所使用之石油系溶劑，107件樣本經GC(FID) 與

GC-MS儀器分析顯示，這些乾洗劑含苯量極低

(0~0.014%)；同時與標準品對照，依其層析圖之

主要成份C9~C14正烷烴可分為五種，A種有66

件，B種10件，C種20件、D種9件及E種2件。其中

正烷烴含量：A、B和C三種各佔總量的26~27%，

D種22%及E種77%。E種的蒸氣樣本更達98%，假

如建議使用E種，則可使乾洗業工作者暴露於揮發

性有機物質下的危險性達最低程度。B種的二甲苯

與1,2,4-三甲基苯芳香烴各佔2%左右。

從以上發現，初步了解台北市乾洗業使用的

乾洗溶劑化學特性；研究結果可作為乾洗業界選

購乾洗油的參考；同時，經由適當的正烷類有機

溶劑混合，或可作為發展或合成更環保更具職業

安全性與更佳洗淨力的--第21世紀新一代Stoddard溶劑。

英文摘要

Long term, chronic exposure to high concentration petroleum series

dry-cleaning solvent will cause dermatitis and central nervous system

depression; Aplastic anemia and leukemia might occur if high

concentration of benzene is not refined. The benzene concentration in our

domestic gasoline is around 2.0%. By means of the GC(FID) and GC-MS

analysis, 107 samples of petroleum series dry-cleaning solvent from the

dry-cleaning stores in Taipei were evaluated. Almost undetectable level of

benzene (0-0.014%) is determined in this study. Other than the non-

detectable benzene, samples can be classified into 5 types according to the

majority components with the authentic standards, i.e. C9~C14 n-alkane

compounds. The 107 samples can thus be grouped as : type A (66)、B

(10)、 C (20)、D (9) and E (2). The n-alkane content varies: type A, B &C

is around 27%, D is 22% and E is 77%. The vapor phase of type E sample

(C9~C14 n-alkane) is even more 98%, might suggest the least hazard of

VOCs expoure in dry-cleaning operation if sample E were used. Around

2% of the aromatic compounds, including xylene, and 1,2,4-

trimethylbenzene were found in type B sample.

From the above findings, the preliminary chemical characteristics of

petroleum series dry-cleaning solvent used in Taipei were unveiled. These

results will be useful for vendor&apos;&apos;s selection in their future solvent purchase.

Also, the more environmental-occupational, and operational solvent--the

21st. century new Stoddard dry-cleaning solvent, might be developed or

synthesized through proper n-alkane series combination.